Serial Tool for the Palm III.  Released under the GNU general public

license, see www.gnu.org for the full terms (Yes it *IS* copyrighted,

but freely redistributable including modifications as long as you

follow the terms of the GNU GPL, or ask me for permission.

May work on other models, but not IrDA without modification.

Notes:

Default baud rate is 38400 (or what this divisor produces under

EcoHack in a different mode - set EcoHack to no serial control and

list it under the apps, or...).  It also defaults to RS232.  It uses

8N1, and I have provided no way of changing it.

The hex buttons enter data.  bs is a backspace.

To set the baud, enter the desired rate and hit Baud.  The computed

(actual) baud rate will be returned.

To switch to the IrDA port, hit IrDA, to go back to the hotsync port,

hit RS232.

To send characters, type in the hex digit pairs and hit Send.

Entering a graffitum will cause the character's hex value to be

entered.

For frequently used messages, enter the hex digits followed by an

extra hex digit and hit M (at the bottom of the digits).  Now when you

use a single digit followed by M, it will recall the entered line,

e.g.  A 1 B 2 C 3 8 M will store A1B2C3 into memory 8, and 8 M will

recall it.  There are only 10 memorys, the digits A-F are special

functions.

To change the system clock frequency (i.e. what EcoHack does), enter

the value and hit Aux.

Memory A returns the UART registers, see the dragonball datasheet and

the source for more details.  (i.e. use M A to recall this).  Memory B

returns the current actual baud rate after a system clock change.

Memory F sends the experimental Ir Remote sequence.

Compilation notes:

This uses the Linux gcc set of pilot development tools.

font is a misnomer since it is technically a set of bitmaps.