.pl110.ff1

.tc 75 0 0 0 0 0 2 0 2 0 2 0 2 0

.te1.ce.e1Introduction.e0

This  describes  a  proposed  standard  MIDI  file format.  MIDI

files contain one or more  MIDI streams, with time information  for each

event.  Song, sequence, and  track structures, tempo and time  signature

information,  are  all  supported.   Track  names  and other descriptive

information may  be stored  with the  MIDI data.   This format  supports

multiple tracks and multiple sequences so that if the user of a  program

which supports multiple  tracks intends to  move a file  to another one,

this format can allow that to happen.

This  spec  defines  the  8-bit  binary  data stream used in the

file. The data  can be stored  in a binary  file, nibbleized, 7-bit-ized

for efficient MIDI transmission,  converted to Hex ASCII,  or translated

symbolically to  a printable  text file.  This spec  addresses what's in

the 8-bit stream.

.te1.ce.e1Sequences, Tracks, Chunks: File Block Structure.e0

Sequence  files  are  made  up  of  chunks.  Each  chunk  has  a

4-character type and a  32-bit length, which is  the number of bytes  in

the chunk. On the Macintosh, data  is passed either in the data  fork of

a file, or on the Clipboard. (The file type on the Macintosh for a  file

in this  format will  be "Midi".)  On any  other computer,  the data  is

simply the  contents of  the file.  This structure  allows future  chunk

types to be  designed which may  easily be ignored  if encountered by  a

program written  before the  chunk type  is introduced.   Your  programs

should expect alien chunks and treat them as if they weren't there.

This proposal defines two types  of chunks:  a header  chunk and

a track chunk. A header  chunk provides a minimal amount  of information

pertaining to the entire MIDI file. A track chunk contains a  sequential

stream of  MIDI data  which may  contain information  for up  to 16 MIDI

channels.  The  concepts  of  multiple  tracks,  multiple  MIDI outputs,

patterns,  sequences,  and  songs  may  all be implemented using several

track chunks.

A MIDI file always starts  with a header chunk, and  is followed

by one or more track chunks.

MThd  <length of header data>

<header data>

MTrk  <length of track data>

<track data>

MTrk  <length of track data>

<track data>

...

Track Data Format (MTrk chunk type)

The MTrk chunk type is where  actual song data is stored. It  is

simply  a  stream  of  MIDI  events  (and  non-MIDI events), preceded by

delta-time values.

Some numbers in MTrk chunks  are represented in a form  called a

variable-length  quantity.  These  numbers  are  represented  7 bits per

byte, most significant bits first. All bytes except the last have bit  7

set, and the last byte has bit  7 clear. If the number is between  0 and

127, it is thus represented exactly as one byte.

Here are some examples of numbers represented as variable-length

quantities:

Number (hex)    Representation (hex)

00000000    00

00000040    40

0000007F    7F

00000080    81 00

00002000    C0 00

00003FFF    FF 7F

00004000    81 80 00

00100000    C0 80 00

001FFFFF    FF FF 7F

00200000    81 80 80 00

08000000    C0 80 80 00

0FFFFFFF    FF FF FF 7F

The  largest  number   which  is  allowed   is  0FFFFFFF  so   that  the

variable-length  representation  must  fit  in  32  bits in a routine to

write  variable-length  numbers.   Theoretically,  larger  numbers   are

possible, but 2 x 108 96ths of a  beat at a fast tempo of 500 beats  per

minute is four days, long enough for any delta-time!

Here is the syntax of an MTrk chunk:

<track data> = <MTrk event>+

<MTrk event> = <delta-time> <event>

<delta-time>  is  stored  as  a  variable-length  quantity.   It

represents the amount of time  before the following event. If  the first

event in  a track  occurs at  the very  beginning of  a track, or if two

events occur simultaneously, a  delta-time of zero is  used. Delta-times

are always present. (Not storing delta-times of 0 requires at least  two

bytes  for  any  other  value,   and  most  delta-times  aren't   zero.)

Delta-time is in some fraction of  a beat (or a second, for  recording a

track with SMPTE times), as specified in the header chunk.

<event> = <MIDI event> | <sysex event> | <meta-event>

<MIDI  event>  is  any  MIDI  channel message. Running status is

used:   status bytes  may be  omitted after  the first  byte. The  first

event in  a file  must specify  status. Delta-time  is not considered an

event itself:  it is an integral part of the specification. Notice  that

running status occurs across delta-times.

<meta-event>  specifies  non-MIDI  information  useful  to  this

format or to sequencers, with this syntax:

FF <type> <length> <bytes>

All meta-events  begin with  FF, then  have an  event type  byte

(which is always less  than 128), and then  have the length of  the data

stored  as  a  variable-length  quantity,  and  then the data itself. If

there is no data, the length is 0. As with sysex events, running  status

is  not  allowed.  As  with  chunks,  future meta-events may be designed

which may not be known  to existing programs, so programs  must properly

ignore  meta-events  which  they  do  not  recognize, and indeed, should

expect  to  see  them.  New  for  0.06:   programs must never ignore the

length of a  meta-event which they  do recognize, and  they shouldn't be

surprised if  it's bigger  than they  expected. If  so, they must ignore

everything  past  what  they  know  about.  However,  they  must not add

anything of their own to the end of a meta-event.

<sysex  event>  is  used  to  specify  a  MIDI  system exclusive

message,  or  as  an  "escape"  to  specify  any  arbitrary  bytes to be

transmitted.   Unfortunately,  some  synthesizer  manufacturers  specify

that their  system exclusive  messages are  to be  transmitted as little

packets.  Each  packet  is  only  part  of  an entire syntactical system

exclusive message, but the times they are transmitted at are  important.

Examples of this are the bytes sent  in a CZ patch dump, or the  FB-01's

"system exclusive  mode" in  which microtonal  data can  be transmitted.

To be able to handle situations like these, two forms of  <sysex  event>

are provided:

F0 <length> <bytes to be transmitted after F0>

F7 <length> <all bytes to be transmitted>

In  both  cases,  <length>   is  stored  as  a   variable-length

quantity.   It  is  equal  to  the  number  of  bytes  following it, not

including itself  or the  message type  (F0 or  F7), but  all the  bytes

which  follow,  including  any  F7  at  the  end which is intended to be

transmitted.  The  first   form,  with  the   F0  code,  is   used   for

syntactically complete  system exclusive  messages, or  the first packet

an a series Q that is,  messages in which the F0 should  be transmitted.

The  second  form  is  used  for  the  remainder of the packets within a

syntactic sysex message, which do not  begin with F0. Of course, the  F7

is not considered part of the system exclusive message. Of course,  just

as in  MIDI, running  status is  not allowed,  in this  case because the

length is  stored as  a variable-length  quantity which  may or  may not

start with bit 7 set.

(New to 0.06)  A syntactic system exclusive message must  always

end with an F7,  even if the real-life  device didn't send one,  so that

you know when you've reached the end of an entire sysex message  without

looking ahead  to the  next event  in the  MIDI file.  This principle is

repeated and illustrated in the paragraphs below.

The vast  majority of  system exclusive  messages will  just use

the F0 format. For instance, the  transmitted message F0 43 12 00  07 F7

would be stored in  a MIDI file as  F0 05 43 12  00 07 F7. As  mentioned

above, it is required  to include the F7  at the end so  that the reader

of the MIDI file knows that it has read the entire message.

For special  situations when  a single  system exclusive message

is split  up, with  parts of  it being  transmitted at  different times,

such as in a Casio CZ  patch transfer, or the FB-01's "system  exclusive

mode", the F7  form of sysex  event is used  for each packet  except the

first. None of  the packets would  end with an  F7 except the  last one,

which must end  with an F7.   There also must  not be any  transmittable

MIDI events in- between the  packets of a multi-packet system  exclusive

message. Here is an  example: suppose the bytes  F0 43 12 00  were to be

sent, followed by a 200-tick delay, followed  by the bytes  43 12 00  43

12 00, followed  by a 100-tick  delay, followed by  the bytes   43 12 00

F7, this would be in the MIDI File:

F0 03 43 12 00

81 48                   200-tick delta-time

F7 06 43 12 00 43 12 00

64                  100-tick delta-time

F7 04 43 12 00 F7

The F7 event  may also be  used as an  "escape" to transmit  any

bytes whatsoever, including real-time bytes, song pointer, or MIDI  Time

Code, which are not permitted normally in this specification. No  effort

should be made to interpret the  bytes used in this way. Since  a system

exclusive  message  is  not  being  transmitted,  it is not necessary or

appropriate to end the F7 event with an F7 in this case.

.te1.ce.e1Header Chunk.e0

The header  chunk at  the beginning  of the  file specifies some

basic information about the data in the file. The data section  contains

three  16-bit  words,  stored  high  byte  first (of course). Here's the

syntax of the complete chunk:

<chunk type> <length> <format> <ntrks> <division>

As described above,  <chunk type> is  the four ASCII  characters

'MThd'; <length> is a 32-bit  representation of the number 6  (high byte

first). The first  word, format, specifies  the overall organization  of

the file. Only three values of format are specified:

0   the file contains a single multi-channel track

1   the file contains one or more simultaneous tracks (or MIDI

outputs) of a sequence

2   the file contains one or more sequentially independent

single-track patterns

The  next  word,  ntrks,  is  the  number of track chunks in the

file.  The  third  word,  division,  is  the  division of a quarter-note

represented by the  delta-times in the  file. (If division  is negative,

it represents the  division of a  second represented by  the delta-times

in the file, so that the track can represent events occurring in  actual

time instead of metrical time.  It is represented in the  following way:

the  upper  byte  is  one  of  the  four  values  -24, -25, -29, or -30,

corresponding to  the four  standard SMPTE  and MIDI  time code formats,

and represents the number of frames per second. The second byte  (stored

positive) is  the resolution  within a  frame: typical  values may  be 4

(MIDI time code  resolution), 8, 10,  80 (bit resolution),  or 100. This

system allows  exact specification  of time-code-based  tracks, but also

allows  millisecond-based  tracks  by  specifying  25  frames/sec  and a

resolution of 40 units per frame.)

Format  0,  that  is,  one  multi-channel  track,  is  the  most

interchangeable representation  of data.  One application  of MIDI files

is  a  simple  single-track  player  in  a  program  which needs to make

synthesizers  make  sounds,  but  which  is  primarily  concerned   with

something  else  such  as  mixers  or  sound  effect  boxes.  It is very

desirable to be able to produce  such a format, even if your  program is

track-based, in order to work  with these simple programs. On  the other

hand, perhaps someone  will write a  format conversion from  format 1 to

format 0 which  might be so  easy to use  in some setting  that it would

save you the trouble of putting it into your program.

Programs  which  support  several  simultaneous tracks should be

able to  save and  read data  in format  1, a vertically one-dimensional

form,  that  is,  as  a  collection  of  tracks.  Programs which support

several independent  patterns should  be able  to save  and read data in

format 2, a horizontally  one-dimensional form. Providing these  minimum

capabilities will ensure maximum interchangeability.

MIDI files can express tempo  and time signature, and they  have

been chosen  to do  so for  transferring tempo  maps from  one device to

another.  For a format 0  file, the tempo will be scattered  through the

track and  the tempo  map reader  should ignore  the intervening events;

for a format 1 file, the tempo map must (starting in 0.04) be stored  as

the first track. It is polite to  a tempo map reader to offer your  user

the ability to make a format 0 file with just the tempo, unless you  can

use format 1.

All MIDI files should specify tempo and time signature. If  they

don't, the time signature is assumed to be 4/4, and the tempo 120  beats

per minute. In format 0, these meta-events should occur at least at  the

beginning  of  the  single  multi-channel  track.  In  format  1,  these

meta-events should be contained in the  first track.  In format 2,  each

of the temporally independent  patterns should contain at  least initial

time signature and tempo information.

We  may  decide  to  define  other  format  IDs to support other

structures. A program reading an  unfamiliar format ID should return  an

error to the user rather than trying to read further.

.te1.ce.e1Meta-Events.e0

A few  meta-events are  defined herein.  It is  not required for

every  program  to  support  every  meta-event.  Meta-events   initially

defined include:

FF 00 02 ssss   Sequence Number

This optional  event, which  must occur  at the  beginning of  a

track,  before  any  nonzero  delta-times,  and before any transmittable

MIDI events,  specifies the  number of  a sequence.  The number  in this

track  corresponds  to  the  sequence  number  in  the  new  Cue message

discussed at the summer  1987 MMA meeting. In  a format 2 MIDI  file, it

is used to identify each "pattern"  so that a "song" sequence using  the

Cue message to  refer to the  patterns. If the  ID numbers are  omitted,

the sequences' locations in order in the file are used as defaults.   In

a format 0 or 1 MIDI file, which only contain one sequence, this  number

should  be  contained  in  the  first  (or  only)  track. If transfer of

several multitrack sequences is required,  this must be done as  a group

of format 1 files, each with a different sequence number.

FF 01 len text  Text Event

Any amount  of text  describing anything.  It is  a good idea to

put a text  event right at  the beginning of  a track, with  the name of

the track, a  description of its  intended orchestration, and  any other

information which  the user  wants to  put there.  Text events  may also

occur at other times in a  track, to be used as lyrics,  or descriptions

of  cue  points.   The  text  in  this  event  should be printable ASCII

characters  for  maximum  interchange.  However,  other  character codes

using the high-order  bit may be  used for interchange  of files between

different  programs  on  the  same  computer  which supports an extended

character set.  Programs on a computer which does not support  non-ASCII

characters should ignore those characters.

(New for  0.06 ).  Meta event  types 01  through 0F are reserved

for various types of text events, each of which meets the  specification

of text events(above) but is used for a different purpose:

FF 02 len text  Copyright Notice

Contains a copyright notice as printable ASCII text. The  notice

should contain the  characters (C), the  year of the  copyright, and the

owner of the copyright. If several pieces of music are in the same  MIDI

file, all  of the  copyright notices  should be  placed together in this

event  so  that  it  will  be  at  the beginning of the file. This event

should be the first event in the first track chunk, at time 0.

FF 03 len text  Sequence/Track Name

If in a format 0 track, or  the first track in a format 1  file,

the name of the sequence. Otherwise, the name of the track.

FF 04 len text  Instrument Name

A description of the type of instrumentation to be used in  that

track. May  be used  with the  MIDI Prefix  meta-event to  specify which

MIDI  channel  the  description  applies  to,  or  the  channel  may  be

specified as text in the event itself.

FF 05 len text  Lyric

A lyric to be sung. Generally, each syllable will be a  separate

lyric event which begins at the event's time.

FF 06 len text  Marker

Normally in a format 0 track,  or the first track in a  format 1

file.  The  name  of  that  point  in  the sequence, such as a rehearsal

letter or section name ("First Verse", etc.).

FF 07 len text  Cue Point

A description of something happening on a film or video screen or  stage

at that point in the  musical score ("Car crashes into  house", "curtain

opens", "she slaps his face", etc.)

FF 2F 00    End of Track

This event  is not  optional. It  is included  so that  an exact

ending point may  be specified for  the track, so  that it has  an exact

length, which is necessary for tracks which are looped or concatenated.

FF 51 03 tttttt     Set Tempo, in microseconds per MIDI quarter-note

This  event  indicates  a  tempo  change. Another way of putting

"microseconds  per  quarter-note"  is  "24ths  of a microsecond per MIDI

clock". Representing tempos  as time per  beat instead of  beat per time

allows  absolutely  exact  long-term  synchronization  with a time-based

sync protocol such as SMPTE time code or MIDI time code. This amount  of

accuracy provided by  this tempo resolution  allows a four-minute  piece

at 120 beats  per minute to  be accurate within  500 usec at  the end of

the piece.  Ideally, these  events should  only occur  where MIDI clocks

would  be  located  Q  this  convention  is intended to guarantee, or at

least   increase   the   likelihood,   of   compatibility   with   other

synchronization devices  so that  a time  signature/tempo map  stored in

this format may easily be transferred to another device.

FF 54 05 hr mn se fr ff SMPTE Offset (New in 0.06 - SMPTE Format

specification)

This event, if present, designates  the SMPTE time at which  the

track chunk is supposed to start. It should be present at the  beginning

of the track,  that is, before  any nonzero delta-times,  and before any

transmittable  MIDI  events.  The  hour  must  be encoded with the SMPTE

format, just as it is in MIDI Time Code.  In a format 1 file, the  SMPTE

Offset must be stored with the tempo  map, and has no meaning in any  of

the other tracks. The ff field contains fractional frames, in 100ths  of

a frame,  even in  SMPTE-based tracks  which specify  a different  frame

subdivision for delta-times.

FF 58 04 nn dd cc bb    Time Signature

The time  signature is  expressed as  four numbers.  nn and dd represent

the  numerator  and  denominator  of  the  time signature as it would be

notated. The  denominator is  a negative  power of  two: 2  represents a

quarter-note,  3  represents  an  eighth-note,  etc.  The  cc  parameter

expresses  the  number  of  MIDI  clocks  in  a  metronome click. The bb

parameter  expresses  the  number  of  notated  32nd-notes  in  a   MIDI

quarter-note (24 MIDI Clocks). This was added because there are  already

multiple programs which allow the user to specify that what MIDI  thinks

of as a quarter-note (24 clocks) is  to be notated as, or related to  in

terms of, something else.

Therefore, the complete event for  6/8 time, where the metronome  clicks

every three eighth-notes, but there  are 24 clocks per quarter-note,  72

to the bar, would be (in hex):

FF 58 04 06 03 24 08

That is, 6/8 time (8 is 2 to  the 3rd power, so this is 06 03),  32 MIDI

clocks per dotted-quarter  (24 hex!), and  eight notated 32nd-notes  per

MIDI quarter note.

FF 59 02 sf mi  Key Signature

sf = -7:  7 flats

sf = -1:  1 flat

sf = 0:  key of C

sf = 1:  1 sharp

sf = 7: 7 sharps

mi = 0:  major key

mi = 1:  minor key

FF 7F len data  Sequencer-Specific Meta-Event

Special  requirements  for  particular  sequencers  may use this

event  type:  the  first  byte  or  bytes  of data is a manufacturer ID.

However, as this is an interchange format, growth of the spec proper  is

preferred to use of this event type.  This type of event may be used  by

a  sequencer  which  elects  to  use  this  as  its  only  file  format;

sequencers  with  their  established  feature-specific  formats   should

probably stick to the standard features when using this format.

.te1.ce.e1Program Fragments and Example MIDI Files.e0

Here are some of the routines to read and write  variable-length

numbers in MIDI Files. These routines  are in C, and use getc  and putc,

which read and  write single 8-bit  characters from/to the  files infile

and outfile.

void	writeVariableLength(dword value,FILE *fp)

{

dword	buffer = value&0x7F;

while((value>>=7)>0){

buffer<<=8,

buffer|=0x80;

buffer+=(value&0x7F);

}

forever{

fputc(buffer,fp);

if(buffer&0x80)

buffer>>=8;

else

break;

}

}

dword	readVariableLength(FILE *fp)

{

dword	rv;

byte	b;

if((rv=fgetc(fp))&0x80){

rv&=0x7F;

do{

rv=(rv<<7)+((b=fgetc(fp))&0x7F);

}while(b&0x80);

}

return rv;

}

As an example,  MIDI Files for  the following excerpt  are shown

below.   First,  a  format  0  file  is  shown,  with  all   information

intermingled; then,  a format  1 file  is shown  with all data separated

into four tracks:  one for  tempo and time signature, and three  for the

notes.  A  resolution of 96  "ticks" per quarter  note is used.   A time

signature of  4/4 and  a tempo  of 120,  though implied,  are explicitly

stated.

The contents of the MIDI stream represented by this example  are

broken down here:

Delta   Event

Time    Code

(dec)   (hex)   Other Bytes (decimal)

Comment

0   FF 58   04 04 02 24 08      	4 bytes: 4/4 time, 24 MIDI

clocks/click, 8 32nd notes/24

MIDI clocks

0   FF 51   03 500000		3 bytes: 500,000 5sec per

quarter-note

0   C0  5           		Ch. 1 Program Change 5

0   C0  5           		Ch. 1 Program Change 5

0   C1  46          		Ch. 2 Program Change 46

0   C2  70          		Ch. 3 Program Change 70

0   92  48  96          		Ch. 3 Note On C2, forte

0   92  60  96          		Ch. 3 Note On C3, forte

96  91  67  64          		Ch. 2 Note On G3, mezzo-forte

96  90  76  32          		Ch. 1 Note On E4, piano

192 82  48  64          		Ch. 3 Note Off C2, standard

0   82  60  64          		Ch. 3 Note Off C3, standard

0   81  67  64          		Ch. 2 Note Off G3, standard

0   80  76  64          		Ch. 1 Note Off E4, standard

0   FF 2F   00      		Track End

The entire  format 0  MIDI file  contents in  hex follow. First,

the header chunk:

4D 54 68 64		MThd

00 00 00 06         chunk length

00 00               format 0

00 01               one track

00 60               96 per quarter-note

Then,  the  track  chunk.  Its  header,  followed  by the events

(notice that running status is used in places):

4D 54 72 6B         	MTrk

00 00 00 3B         	chunk length (59)

Delta-time  Event           Comments

00  FF 58 04 04 02 18 08    time signature

00  FF 51 03 07 A1 20       tempo

00  C0 05               Ch. 1 Program Change 05

00  C1 2E               Ch. 2 Program Change 2E

00  C2 46

00  92 30 60            Ch. 3 Note On

00  3C 60               running status

60  91 43 40

60  90 4C 20

81 40   82 30 40            two-byte delta-time

00  3C 40               running status

00  81 43 40

00  80 4C 40

00  FF 2F 00        end of track

A format  1 representation  of the  file is  slightly different.

Its header chunk:

4D 54 68 64             MThd

00 00 00 06         chunk length

00 01               format 1

00 04               four tracks

00 60               96 per quarter-note

First, the track chunk  for the time signature/tempo  track. Its

header, followed by the events:

4D 54 72 6B         MTrk

00 00 00 14         chunk length (20)

Delta-time  Event           Comments

00  FF 58 04 04 02 18 08        time signature

00  FF 51 03 07 A1 20       tempo

83 00   FF 2F 00        end of track

Then,  the  track  chunk  for  the  first  music track. The MIDI

convention for note on/off running status is used in this example:

4D 54 72 6B         MTrk

00 00 00 10         chunk length (16)

Delta-time  Event           Comments

00  C0 05

81 40   90 4C 20

81 40   4C 00               Running status: note on, vel = 0

00  FF 2F 00        end of track

Then, the track chunk for the second music track:

4D 54 72 6B         MTrk

00 00 00 0F         chunk length (15)

Delta-time  Event   Comments

00  C1 2E

60  91 43 40

82 20   43 00               running status

00  FF 2F 00        end of track

Then, the track chunk for the third music track:

4D 54 72 6B         MTrk

00 00 00 15     chunk length (21)

Delta-time  Event       Comments

00  C2 46

00  92 30 60

00  3C 60               running status

83 00   30 00               two-byte delta-time, running status

00  3C 00               running status

00  FF 2F 00        end of track

.te1.ce.e1MIDI Transmission of MIDI Files.e0

Since it  is inconvenient  to exchange  disks between  different

computers, and  since many  computers which  will use  this format  will

have a  MIDI interface  anyway, MIDI  seems like  a perfect  way to send

these  files  from  one  computer  to  another.   And, while we're going

through all the trouble  to make a way  of sending MIDI Files,  it would

be nice if  they could send  any files (like  sampled sound files,  text

files, etc.)

.te1.ce.e1Goals.e0

The transmission  protocol for  MIDI files  should be reasonably

efficient,  should  support  fast  transmission  for computers which are

capable  of  it,  and  slower  transmission  for  less powerful ones. It

should not be impossible to convert a MIDI File to or from an  arbitrary

internal representation on  the fly as  it is transmitted,  but, as long

as  it  is  not  too  difficult,  it  is very desirable to use a generic

method so that any file type could be accommodated.

To make the protocol  efficient, the MIDI transmission  of these

files will take groups of seven  8-bit bytes and transmit them as  eight

7-bit MIDI data bytes.  This is certainly in  the spirit of the  rest of

this  format  (keep  it  small,  because  it's  not that hard to do). To

accommodate  a  wide  range  of  transmission  speeds,  files  will   be

transmitted  in  packets  with  acknowledge  --  this  allows data to be

stored to  disk as  it is  received. If  the sender  does not  receive a

response from a  reader in a  certain amount of  time, it can  assume an

open-loop situation, and then just continue.

The last edition of MIDI Files contained a specialized  protocol

for sending just MIDI Files.  To meet a deadline, unfortunately  I don't

have time right now to propose a new generalized protocol. This will  be

done within  the next  couple of  months. I  would welcome any proposals

anyone else has,  and would direct  your attention to  the proposal from

Ralph Muha  of Kurzweil,  available in  a recent  MMA bulletin, and also

directly