----------104F00-----------------------------

INT 10 - VESA SuperVGA BIOS - GET SuperVGA INFORMATION

AX = 4F00h

ES:DI -> 256-byte buffer for SuperVGA information (see below)

Return: AL = 4Fh function supported

AH = status

00h successful

01h failed

SeeAlso: AX=4F01h

Format of SuperVGA information:

Offset  Size      Description

00h   4 BYTEs    signature ('VESA')

04h    WORD      VESA version number

06h    DWORD     pointer to OEM name

0Ah   4 BYTEs    capabilities

0Eh    DWORD     pointer to list of supported VESA and OEM video modes

12h    WORD      total on-board memory installed (in 64K blocks)

14h   238 BYTEs  reserved

----------104F01-----------------------------

INT 10 - VESA SuperVGA BIOS - GET SuperVGA MODE INFORMATION

AX = 4F01h

CX = SuperVGA video mode

ES:DI -> 256-byte buffer mode information (see below)

Return: AL = 4Fh function supported

AH = status

00h successful

01h failed

SeeAlso: AX=4F00h,AX=4F02h

Format of mode information:

Offset Size   Description

00h   WORD   mode attributes

bit 0: mode supported

bit 1: optional information available

bit 2: BIOS output supported

bit 3: set if color, clear if monochrome

bit 4: set if graphics mode, clear if text mode

02h   BYTE   window A attributes

bit 0: exists

bit 1: readable

bit 2: writable

bits 3-7 reserved

03h   BYTE   window B attributes (as for window A)

04h   WORD   window granularity in K

06h   WORD   window size in K

08h   WORD   start segment of window A

0Ah   WORD   start segment of window B

0Ch   DWORD -> FAR window positioning function (equivalent to AX=4F05h)

10h   WORD   bytes per scan line

---remainder is optional for VESA modes, needed for OEM modes---

12h   WORD   width in pixels

14h   WORD   height in pixels

16h   BYTE   width of character cell in pixels

17h   BYTE   height if character cell in pixels

18h   BYTE   number of memory planes

19h   BYTE   number of bits per pixel

1Ah   BYTE   number of banks

1Bh   BYTE   memory model type

1Ch   BYTE   size of bank in K

----------104F02-----------------------------

INT 10 - VESA SuperVGA BIOS - SET SuperVGA VIDEO MODE

AX = 4F02h

BX = mode

bit 15 set means don't clear video memory

Return: AL = 4Fh function supported

AH = status

00h successful

01h failed

SeeAlso: AX=4F01h,AX=4F03h

Values for VESA video mode:

00h-FFh OEM video modes (see AH=00h)

100h 640x400x256

101h 640x480x256

102h 800x600x16

103h 800x600x256

104h 1024x768x16

105h 1024x768x256

106h 1280x1024x16

107h 1280x1024x256

108h 80x60  text

109h 132x25 text

10Ah 132x43 text

10Bh 132x50 text

10Ch 132x60 text

110h 640x480x32768    15-bit hi-color

111h 640x480x65536    16-bit hi-color

112h 640x480x16777216 24-bit true-color

113h 800x600x32768    15-bit hi-color

114h 800x600x65536    16-bit hi-color

----------104F03-----------------------------

INT 10 - VESA SuperVGA BIOS - GET CURRENT VIDEO MODE

AX = 4F03h

Return: AL = 4Fh function supported

AH = status

00h successful

01h failed

BX = video mode (see AX=4F02h)

SeeAlso: AX=4F02h

----------104F04-----------------------------

INT 10 - VESA SuperVGA BIOS - SAVE/RESTORE SuperVGA VIDEO STATE

AX = 4F04h

DL = subfunction

00h get state buffer size

Return: BX = number of 64-byte blocks needed

01h save video states

ES:BX -> buffer

02h restore video states

ES:BX -> buffer

CX = flags for states to save/restore

bit 0: video hardware state

bit 1: video BIOS data state

bit 2: video DAC state

bit 3: SuperVGA state

Return: AL = 4Fh function supported

AH = status

00h successful

01h failed

----------104F05-----------------------------

INT 10 - VESA SuperVGA BIOS - CPU VIDEO MEMORY CONTROL

AX = 4F05h

BH = subfunction

00h select video memory window

DX = window address in video memory (in granularity units)

01h get video memory window

Return: DX = window address in video memory (in gran. units)

BL = window number

00h window A

01h window B

Return: AL = 4Fh function supported

AH = status

00h successful

01h failed

SeeAlso: AX=4F06h,AX=4F07h,AX=7000h/BX=0004h

----------104F06-----------------------------

INT 10 - VESA SuperVGA BIOS 1.1 - GET/SET LOGICAL SCAN LINE LENGTH

AX = 4F06h

BL = function

00h set scan line length

CX = desired width in pixels

01h get scan line length

Return: AL = 4Fh if function supported

AH = status

00h successful

01h failed

BX = bytes per scan line

CX = number of pixels per scan line

DX = maximum number of scan lines

Notes: 	if the desired width is not achievable, the next larger width will be set

the scan line may be wider than the visible area of the screen

this function is valid in text modes, provided that values are

multiplied by the character cell width/height

SeeAlso: AX=4F01h,AX=4F05h,AX=4F07h

----------104F07BH00-------------------------

INT 10 - VESA SuperVGA BIOS 1.1 - GET/SET DISPLAY START

AX = 4F07h

BH = 00h (reserved)

BL = 00h set display start

CX = leftmost displayed pixel in scan line

DX = first displayed scan line

= 01h get display start

Return: BH = 00h

CX = leftmost displayed pixel in scan line

DX = first displayed scan line

Return: AL = 4Fh if function supported

AH = status

00h successful

01h failed

Note: this function is valid in text modes, provided that values are

multiplied by the character cell width/height

SeeAlso: AX=4F01h,AX=4F05h,AX=4F06h

----------104FFF-----------------------------

INT 10 - VESA SuperVGA BIOS - Everex - TURN VESA ON/OFF

AX = 4FFFh

DL = new state (00h off, 01h on)