Tseng Super VGA

ET3000-AX   512k 8/16 bit Main chip

-BX   512k  8 bit

-Bp   256k  8 bit  Basic chip

ET4000      1M 8/16 bit

Registers:

102h: Microchannel Setup Control

bit 0  Disable Card if set

3C0h index 16h: ATC Miscellaneous

(Write data to 3C0h, Read from 3C1h  (May not be needed))

bit 4,5  High resoloutionn timings.

3C3h (R/W): Microchannel Video Subsystem Enable Register:

bit 0  Enable Microchannel VGA if set

3C4h index  6  (R/W): Zoom Control   (ET3000 Only)

bit 0-2  Yzoom factor 0=1:1

4-6  Xzoom factor 0=1:1

7  Hardware zoom enabled if set

3C4h index  7  (R/W): TS Auxiliary Mode

bit 0  (ET3000 Only) Switch Normal Window if set (Text/Graph)

1  (ET3000 Only) Switch Zoom Window if set (Text/Graph)

2  (ET3000 Only) Switch Split Window if set (Text/Graph)

4  (ET3000 Only) If set 8 simultaneous fonts are enabled,

using bit 3,4,6 of each attribute byte

to select the font.

3,5  Rom Bios Enable/Disable:

0 0  C000-C3FF Enabled

0 1  Rom disabled

1 0  C000-C5FF,C680-C7FF Enabled

1 1  C000-C7FF Enabled

6  MCLK/2 if set

7  VGA compatible if set EGA else.

3CBh (R/W): PEL Address/Data

3CDh (R/W): Segment Select

bit 0-2  (ET3000) 64k Write bank nr

3-5  (ET3000) 64k Read bank nr

6-7  (ET3000) Segment Configuration.

0  128K segments

1   64K segments

2  1M linear memory

0-3  (ET4000) 64k Write bank nr (0..15)

4-7  (ET4000) 64k Read bank nr (0..15)

3d4h index 1Bh (R/W): X Zoom Start Address       (ET3000 Only)

bit 0-7  Offset of Zoom window start in character clocks

(8 pixels) from left edge

3d4h index 1Ch (R/W): X Zoom End                 (ET3000 Only)

bit 0-7  Offset of Zoom window end in char clocks from left edge

3d4h index 1Dh (R/W): Y Zoom Start Address       (ET3000 Only)

bit 0-7  Start line of zoom window bit 0-7

3d4h index 1Eh (R/W): Y Zoom End Address         (ET3000 Only)

bit 0-7  (ET3000 Only) End line of zoom window bit 0-7

3d4h index 1Fh (R/W): Y Zoom Start and End High  (ET3000 Only)

bit 0-2  End line of zoom window bit 8-10

3-5  Start line of zoom window bit 8-10

3d4h index 20h (R/W): Zoom Start Address Low     (ET3000 Only)

bit 0-7  Zoom Start Address bit 0-7

Adress of Zoom data

3d4h index 21h (R/W): Zoom Start Address Middle  (ET3000 Only)

bit 0-7  Zoom Start Address bit 8-15

3d4h index 23h (R/W): Extended start ET3000      (ET3000 Only)

bit 0  Cursor start adress bit 16

1  Display start adress bit 16

2  Zoom start adress bit 16

3d4h index 24h (R/W): Compatability Control

bit 0  Enable Clock Translate

1  Additional Master Clock Select

2  Enable tri-state for all output pins

3  Enable input A8 of 1MB DRAMs

4  Reserved

5  Enable external ROM CRTC translation

6  Enable Double Scan and Underline Attribute

7  CGA/MDA/Hercules

3d4h index 25h (R/W): Overflow High ET3000       (ET3000 Only)

bit 0  Vertical Blank Start bit 10

1  Vertical Total Start bit 10

2  Vertical Display End bit 10

3  Vertical Sync Start bit 10

4  Line Compare bit 10

5-6  Reserved

7  Vertical Interlace if set

3d4h index 32h (R/W): RAS/CAS Video Config

Ram timing, System clock and Ram type. Sample values:

00h  VRAM  80nsec

09h  VRAM 100nsec

00h  VRAM  28MHz

08h  VRAM  36MHz

70h  DRAM  40MHz

3d4h index 33h (R/W): Extended start ET4000  (ET4000 Only)

bit 0-1  Display start adress bits 16-17

2-3  Cursor start adress bits 16-17

Can be used to ID ET4000

3d4h index 34h (R/W): Compatibility Control Register

bit 3  if set Video Subsystem Enable Register at 46E8h

else at 3C3h.

3d4h index 35h (R/W): Overflow High ET4000   (ET4000 Only)

bit 0  Vertical Blank Start Bit 10

1  Vertical Total Bit 10

2  Vertical Display End Bit 10

3  Vertical Sync Start Bit 10

4  Line Compare Bit 10

5  Gen-Lock Enabled if set (External sync)

6  Read/Modify/Write Enabled if set

Currently not implemented.

7  Vertical interlace if set

3d4h index 36h (R/W): Video System Configuration 1

bit 3  16 bit wide fonts if set, else 8 bit wide

4  (ET4000 Only) Linear adressing if set

Video Memory is mapped as a 1 Meg block above 1MB.

6  16 bit data path (video memory) if set

7  16 bit data (I/O operations) if set

3d4h index 37h (R/W): Video System Configuration 2

bit 0-1  Memory size  0,1=256k, 2=512k, 3=1M   ???

3  Clear if 64kx4 RAMs                   ???

if set RAM size = (bit 0-1)*256k

else RAM size = (bit 0-1)* 64k

4    16 bit ROM access if set

5    Memory bandwidth (0 better than 1) ???

7    VRAM installed if set DRAM else.

bit 6?   Block read ahead (BRA)  if clear

46E8h (R):  Video Subsystem Enable Register

bit 3  Enable VGA if set

3C4h index 05 used.

3CBh used.   PEL Add/Dat. ??

3CEh index 0Dh-0Eh used.

Bank Switching:

64k banks are selected by the Segment Select Register at 3CDh.

Both a Read and a Write segment can be selected.

Hardware Zoom (ET3000 Only).

The ET3000 can zoom a part of display memory in a window.

The display memory position and window position are selected

by 3d4h index 1Bh to 21h.

Identify Tseng Chipset:

old:=port[$3CD];

port[$3cd]:=$55;

new:=port[$3cd];

port[$3cd]:=old;

if new=$55 then Tseng Chip !!!

Identify Tseng 4000:

if odd(port[$3cc]) then base:=$3d4 else base:=$3b4;

port[base]:=$33;old_value:=port[base+1];

new_value:=old_value xor 15;

port[base+1]:=new_value;

value:=port[base+1];

port[base+1]:=old_value;

if value=new_value then ET4000 chip !!!

Video Modes:

8   T   132   25   2                   (STB only)

Ah  T   132   44   2

18h  T   132   44   4  (8x8)   B000

19h  T   132   25   4  (9x14)  B000

1Ah  T   132   28   4  (9x13)  B000

22h  T   132   44  16  (8x8)

23h  T   132   25  16  (8x14)

24h  T   132   28  16  (8x13)

25h  G   640  480  16  planar

26h  T    80   60  16  (8x8)

27h  G   720  512  16  planar           (Tseng recomanded, few boards)

29h  G   800  600  16  planar

2Ah  T   100   40  16  planar

2Dh  G   640  350 256  packed

2Eh  G   640  480 256  packed

2Fh  G   640  400 256  packed           (ET4000 Only)

2Fh  G   720  512 256  packed           (Tseng recomanded, few boards)

30h  G   800  600 256  packed

36h  G   960  720  16  planar           (STB only)

37h  G  1024  768  16  planar

38h  G  1024  768 256  packed           (ET4000 Only)

3Dh  G  1280 1024  16  planar           (newer ET4000s)

See Hicolor.doc for information about the Sierra HiColor RAMDAC.