Paradise/Western Digital Super VGA Chips.

Max mem   Max 16col  256 col

PVGA1A     512k/1M     800x600  640x480

WD90C00    512k/1M    1024x768  800x600   also known as PVGA1B

WD90C10    256k        640x480  320x200

WD90C11    512k       1024x768  800x600

94h (W): Setup Register     (Micro Channel)

102h (R/W): VGA Sleep Register    (setup mode only)

bit 0  VGA enabled if set.

1-7  Reserved.

Note: this register can only be accessed in setup mode.

3C3h (W): Global Enable Register

bit 0  VGA Motherboard Enable if set

3C4h index  3  (R/W): Reconfigured Character Map Select

bit 0-2  Offset of character map in plane 2/3 in 8k blocks

3-4  Character map select from plane:

0: plane 2

1: plane 2 if attr. bit 3 set, plane 3 else

2: plane 3 if attr. bit 3 set, plane 2 else

3: plane 3

3C4h index  7  (R/W): Display Configuration Status (WD90C1x Only)

bit 0  (R) Monitor type

1  (R) EGA emulation if set

2  (R) 6845 compatibility if set

3  (R) Color mode if set

4-7  Scratch Pad

3C4h index 10h (R/W):                              (WD90C11 Only)

bit 2  (WD90C11 Only)

3C4h index 11h (R/W): System Control Interface     (WD90C1x Only)

bit 0  (WD90C1x Only) 16 bit access to CRTC, Graphics

Controller and Sequencer Registers if set

1  (WD90c1x Only) 16 bit access to Attribute Registers if set.

If so 3C0h is index and 3C1h is data port.

2  Enable Write buffer if set

3-4  Enhanced speed depending of host bus speed:

0 for 12MHz, 1 for 10 MHz, 2,3 for 8 MHz

5  Enhanced text mode operations if set

6  Enhanced operation on blank lines if set

7  (WD90C1x Only) If set Bank reg PR0A is used for read cycles

and PR0B for write cycles, else PR0B is used for A000h-A7FFh

and PR0A for A800h-AFFFh.

3C4h index 12h (R/W): Miscellaneous Control #4      (WD90C1x Only)

bit 6  Enable Cursor Blinking if set

7  Enable External Sync

3CEh index  9  (R/W): PR0A Address Offset A

bit 0-6  Added to bit 12-18 of video memory address if

Bank A selected. (Reads if 3C4h index 11h bit 7 set,

A800h-AFFFh if 3CEh index Bh bit set and 64k config

B000h-BFFFh if 128k config. A000h-AFFFh else).

3CEh index  Ah (R/W): PR0B Address Offset B

bit 0-6  Added to bit 12-18 of video memory address if

Bank B selected. (Writes if 3C4h index 11h bit 7 set,

A000h-A7FFh if 3CEh index Bh bit 3 set and 64k config,

A000h-AFFFh if 128k config).

3CEh index  Bh (R/W): PR1 Memory Size

bit 0  Enable on card ROM if set

1  Select 16 bit ROM if set

2  Select 16 bit Memory  if set

3  PROB enabled if set, else use PR0A for both banks

4-5  Memory Map.  0=A000-BFFFh standard VGA,

1=0-3FFFFh, 2=0-7FFFFh and 3=0-FFFFFh

6-7  Installed memory:

0  256k VGA standard

1  256k PVGA bank switching

2  512k do

3    1M do

3CEh index  Ch (R/W): PR2 Video Configuration

bit 0  Force VCLK (overrides SQE1 bit 3).

1  Third Clock Select Line VCLK2

2,5  Character Map Select/Underline

0  Characters are taken from plane 2

1  do, but characters are underlined if

bit 0 of the characters attribute is set.

2  Characters are taken from plane 3.

3  If bit 3 of the characters attribute is set the

character is taken from plane 3, else from plane 2.

3,4  Character Clock Period Control

0  IBM VGA character Clock (8 or 9 dots wide).

1  7 dots (used for 132 character modes).

2  9 dots

3  10 dots

Selecting 10 dots/character modifies the horizontal PEL

panning register (3C0h index 13h).

5  Character Map Select.

6  6845 Compatibility (6845 if set, EGA/VGA if clear).

7  AT&T/M24 Mode Enable, 400 line enable

3CEh index  Dh (R/W): PR3 CRTC Control

bit 0  Lock Vertical Timing

1  Lock Prevention. Prevent locking through the Vertical

Retrace register (3d4h index 11h bit 7).

2  Multiply Cursor Start, Cursor Stop, Preset Row Scan

and Maximum Scan Line registers by 2

3-4  bit 16,17 of CRTC Display Start Address

5  Lock Horizontal Timing

6  Lock HSYNC Polarity

7  Lock VSYNC Polarity

3CEh index  Eh (R/W): PR4 Video Control

bit 0  Extended 256 color Shift register Control. Configures

the video shift registers for extended 256 color mode.

1  EGA compatibility. Disables reads of all registers

that are writeonly on the EGA, and of PR0-5.

2  Lock internal Palette and Overscan registers

3  Override CGA Enable Video bit.

Overrides the CGA "enable video" bit 3 of 3D8h

4  Tri-state Memory Control Outputs

5  Tri-state Video outputs

6  PCLK=VCLK

7  BLNK/Display Enable

3CEh index  Fh (R/W): PR5 General Purpose Status and PR Register

bit 0-2  Extended registers PR0-4 Unlocked if set to 5

4  Read protect PR0-4 if set.

7  Multisync if set, fixed frequency else.

3d4h index 29h (R/W): PR10 Unlock PR11-17 Registers (WD90Cxx Only)

bit 0-2  Write of PR11-18 enabled if set to 5

3,7  Read of PR11-18 enabled if 3 clear and 7 set

3d4h index 2Ah (R/W): PR11 EGA Switches             (WD90Cxx Only)

bit 0  Lock 8/9 Dot Character Clock.

Inhibits writes to 3C4h index 1 bit 0.

1  Lock Graphics and Sequencer Screen Control

Inhibits write to 3CEh index 5 bit 5-6, 3C4h index 1

bit 2-5 and 3C4h index 3 bit 0-5.

2  Lock Clock Select

3  EGA Emulation on Analog Display

4-7  EGA Switch 1-4

3d4h index 2Bh (R/W): PR12 Scratch Pad Register     (WD90Cxx Only)

bit 0-7  Reserved

3d4h index 2Ch (R/W): PR13 Interlace H/2 Start      (WD90Cxx Only)

bit 0-7  Interlace H/2 Start. Adjusts Horizontal Sync

for interlace.

3d4h index 2Dh (R/W): PR14 Interlace H/2 End        (WD90Cxx Only)

bit 0-4  Interlace H/2 End. Adjusts horizontal Sync width

5  Interlace enable

6  Vertical Double Scan for EGA on PS/2 Display

7  Enable Vertical retrace interrupts if set

3d4h index 2Eh (R/W): PR15 Miscellaneous Control #1 (WD90Cxx Only)

bit 0  Disable borders if set

1  Select Display Enable Timing

2  Enables page mode addressing in alpha modes if set

3  Interlace comp. with 8514/A timing if set.

4  VCLK=MCLK. If set MCLK is used for all video timings.

5  VCLK1,VLKC2 Latched Outputs.

6  Low VCLK. Set if VCLK is much slower than MCLK.

7  Enable read of 46E8h

3d4h index 2Fh (R/W): PR16 Miscellaneous Control #2 (WD90Cxx Only)

bit 1  Standard VGA memory if set

3-4  bit 16,17 of CRTC Address Counter Offset

5-6  CRTC Address Counter Width

0= 256k, 1=128K, 2=64K

3d4h index 30h (R/W): PR17 Miscellaneous Control #3 (WD90Cxx Only)

bit 0  Maps out ROM at C600h-C67Fh if set

2  (WD90C1x Only) Enables 64k ROM at C000h-CFFFh if set

3  (WD90C1x Only) Maps out ROM at C600h-C6FFh if set

46E8h (R/W): Global Enable Register           (AT only)

bit 0-2  BIOS ROM page select (8 pages of 4K each).

3  Adapter VGA enabled if set

4  If set the VGA is in Setup mode, where 102h and 46E8h

are the only ports responding.

Wake Up port at 102h responds only if this bit set.

Note: This register is also addressed at 56E8h, 66E8h and 76E8h.

Register only readable if 3d4h index 2Eh bit 7 set

3C4h index 6 used in WD90C1x.

3d4h index 31h-3fh used in WD90C1x.

Bank Switching:

Bank switching happens by adding the bank register to bit 12-19

of the address, resulting in the window starting on a 4k boundry.

Two different bank registers can be used either as Read/Write

banks (WD90C1x) or as two different windows each 32k/64k big.

Memory:

$C000:$09  string   BIOS date

$C000:$35  1 byte   BIOS version

$C000:$7D  4 bytes  $3ED414756   ('VGA=')

ID Paradise/WD Super VGA Chip set:

if meml[$c000:$7d]=$3d414756 ('VGA=') then PARADISE_CHIP !!!

ID Which Paradise/WD Chip:

if odd(port[$3cc]) then base:=$3d4 else base:=$3b4;

port[base]:=$2b;   old_value:=port[base+1];

port[base+1]:=$aa; new_value:=port[base+1];

port[base+1]:=old_value;

if new_value <> $aa then PVGA1A_Chip !!!!

else

port[$3C4]:=$12; old_value:=port[$3C5];

port[$3C5]:=old_value and $bf; new_1:=port[$3C5] and 64;

port[$3C5]:=old_value or  $40; new_2:=port[$3C5] and 64;

port[$3C5]:=old_value;

if (new_1 <>0) or (new_2 =0) then WD90C00_Chip

else

port[$3C4]:=$10; old_value:=port[$3C5];

port[$3C5]:=old_value and $fb; new_1:=port[$3C5] and 4;

port[$3C5]:=old_value or    4; new_2:=port[$3C5] and 4;

port[$3C5]:=old_value;

if (new_1 <>0) or (new_2 =0) then WD90C10_Chip

else WD90C11_Chip;

Video Modes:

54h  T   132   43  16  (7x9 or 8x9)

55h  T   132   25  16  (7x9 or 8x9)

56h  T   132   43   4  (7x9 or 8x9)

57h  T   132   25   4  (7x9 or 8x9)

58h  G   800  600  16  planar

59h  G   800  600   2  odd/even  **** See note

5Ah  G  1024  768   2  odd/even  **** See note

5Bh  G  1024  768   4  packed    **** See note

5Ch  G   800  600 256  packed    WD90C11 Only

5Dh  G  1024  768  16  planar

5Eh  G   640  400 256  packed

5Fh  G   640  480 256  packed

Mode 5Bh  1024x768 4 color.

2 bit per pixel packed mode.

Pixels start in bit 6-7.

Mode 59h   800x600 2 color and

Mode 5Ah  1024x768 2 color.

One bit per pixel odd/even mode

Pixels 0-7 are in plane 0, 8-15 in plane 1.

Bios extensions:

----------10007E-----------------------------

INT 10 - VIDEO - Paradise VGA, AT&T VDC800 - SET SPECIAL MODE

AX = 007Eh

BX = horizontal dimension of the desired mode

CX = vertical dimension of the desired mode

Both BX,CX in dots for graph modes, chars for alpha modes

DX = Number of colors desired (0000h for monochrome)

Return: BH = 7Eh if successful (Paradise VGA)

AL = 7Eh if successful (AT&T VDC800)

----------10007F-----------------------------

INT 10 - VIDEO - Paradise ....   EXTENDED MODE SET

AX = 007Fh

BH = 00h   set VGA operation

BH = 01h   set non-VGA operation

color modes (0,1,2,3,4,5,6) will set non-VGA CGA operation

monochrome mode 7 will set non-VGA MDA/Hercules operation

BH = 02h   Query mode status

Return: BL = 00h if operating in VGA mode, 01h if non-VGA mode

CH = total video RAM size in 64k byte units

CL = video RAM used by the current mode

BH = 03h   Lock current mode. Allows current mode

(VGA or non-VGA) to survive re-boot.

BH = 04h   Set non-VGA MDA/Hercules Mode

BH = 05h   Set non-VGA CGA Mode

BH = 06h   Set VGA Mono Mode

BH = 07h   Set VGA Color Mode

BH = 09h..0Fh   Write Paradise Register

BL = New value of PR register

BH = 19h..1Fh   Read Paradise Register

Return: BL = Value of register

BH: (Read) BH: (Write)   Register

19h        09h          PR0A     Port 3CEh index 09h

1Ah        0Ah          PR0B     Port 3CEh index 0Ah

1Bh        0Bh          PR1      Port 3CEh index 0Bh

1Ch        0Ch          PR2      Port 3CEh index 0Ch

1Dh        0Dh          PR3      Port 3CEh index 0Dh

1Eh        0Eh          PR4      Port 3CEh index 0Eh

1Fh        ---          PR5      Port 3CEh index 0Fh

BH = 20h   Emulate EGA with Analog Monitor   (WD90Cxx Chips Only)

BL=EGA Switches in Bit 0-3

BH = 29h..30h   Read Paradise Register       (WD90Cxx Chips Only)

Return: BL = Value of register

BH:    Register:

29h   PR10     Port 3CEh index 29h

2Ah   PR11     Port 3CEh index 2Ah

2Bh   PR12     Port 3CEh index 2Bh

2Ch   PR13     Port 3CEh index 2Ch

2Dh   PR14     Port 3CEh index 2Dh

2Eh   PR15     Port 3CEh index 2Eh

2Fh   PR16     Port 3CEh index 2Fh

30h   PR17     Port 3CEh index 30h

Return: AL = 7Fh  If succesful  (AT&T VDC600)

BH = 7Fh  If valid call (Paradise/Western Digital)

----------10007F-----------------------------

INT 10 - VIDEO - Paradise VGA, AT&T VDC600 - EXTENDED FUNCTIONS

AX = 007Fh

BH = function

00h	 set VGA operation

01h	 set non-VGA operation

color modes (0,1,2,3,4,5,6) will set non-VGA CGA operation

monochrome mode 7 will set non-VGA MDA/Hercules operation

02h	 query mode status

Return: BL = 00h if operating in VGA mode, 01h if non-VGA mode

CH = total video RAM size in 64k byte units

CL = video RAM used by the current mode

03h	 lock current mode

allows current mode (VGA or non-VGA) to survive re-boot

04h	 enter CGA mode (AT&T VDC600 only)

05h	 enter MDA mode (AT&T VDC600 only)