Ahead V5000 version A & B.

Allows up to four boards in one system.

103h (R/W): Multiple Chip ID Register

bit 0-3  Must match PowerUp register (3CEh index 1Fh bits 4-7).

3C2h (W): Misc Output Register

bit 5  Bit 0 of bank register.         (Ahead A)

Note: This register can be read at 3CCh.

3CEh index  Ch (R/W): mode

bit 0-1  Misc control. 0=standard text mode, 1=enable

8 simultaneous fonts  2&3 reserved.

2  Reserved

3  High speed sequencer enable

4  16 bit memory access enable

5  Enhanced mode enable

6-7  Emulation mode. 0=VGA, 1=EGA, 2=Hercules and 3=CGA

3CEh index  Dh (R/W): Segment            (Different for Ahead A and B)

bit 0-2  (Ahead A) Bank No. bit 1-3

Bit 0 is in 3C2h bit 5.

0-3  (Ahead B) Read Bank No.

4-7  (Ahead B) Write Bank No.

3CEh index  Eh (R/W): Clock

bit 1-3  Reserved

4-7  Divide input clock 0-3 by 2.

3CEh index  Fh (R/W): Master Enable Register

bit 0-3  Chip version number  (Read Only)

0=Ahead A, 1=Ahead B

5  Enable extended registers if set

3CEh index 10h (R/W): Trap

bit 0  Enable CRTC access

1  Enable 6845 access

2  Enable CRTC access to cause trap

3  Enable 3B8h, 3BFh to cause trap

4  Enable 3D8h, 3D9h to cause trap

5  Enable 3Cxh to cause traps

7  Select 6845 as CRT controller

3CEh index 11h (R/W): Trap source

bit 0  3Dxh

1  3B5h, 3D5h

2  3B8h, 3D8h

3  3D9h

4  3BFh

5  3Cxh

6-7  Reserved

3CEh index 12h (R/W): Attribute

bit 0-5  Reserved

6  Lock VGA internal palette

7  Enable CGA palette when in CGA mode

3CEh index 13h (R/W): Diagnostics

bit 0-7  Reserved

3CEh index 14h (R/W): Lock

bit 0  Lock Sync polarity in 3C2h bit 6,7.

1  Lock CRTC horizontal timing

2  Lock CRTC vertical timing

3  Lock CRTC index 9        !Hmm.

4  Lock CRTC index 9

5  Lock CRTC index 0Ah, 0Bh

6  Lock CRTC index 13h

7  Lock Clock select in 3C2h bit 2,3.

3CEh index 15h (R): 3B8/3D8 Readback

bit 0-7  Mono/CGA register 3B8h/3D8h readback value

3CEh index 16h (R): 3BF/3D9 Readback

bit 0-5  CGA Register 3D9h readback

6-7  Mono register 3BFh bit 0-1 Readback.

3CEh index 17h (R/W): Miscellaneous

bit 0  Must be 1

1  Must be 0

2-7  Reserved

3CEh index 1Ch (R/W): CRTC Control

bit 0-1  Bit 16-17 of CRTC start address

2-3  0=normal, 3=Interlaced, 1&2 reserved.

3CEh index 1Dh (R/W): Control

bit 0-7  Reserved

3CEh index 1Eh (R/W): Scratch

bit 0-7  Used by BIOS for flags

3CEh index 1Fh (R): PowerUp

bit 0-1  Memory type. 0=2x44256 (256k), 1=4 or 16 x44256 (512K/2M),

2=8 or 16 x4464 (256K or 512K), 3=8x44256 (1M)

2  0 for 24k BIOS, 1 for 32k BIOS.

3  16 bit BIOS

4-7  Multiple Chip ID

0= ID 0  BIOS Enabled, 1=ID 1  BIOS Enabled

2..15  ID 2..15  Bios Disabled.

46E8h (R/W): Setup Control Register

bit 3  0 for VGA disabled, 1 for enabled

4  0 for Setup mode, 1 for normal mode.

5-7  Reserved

Bank switching:

The Ahead A has one bank register with bit 0 in 3C2h bit 5 and

bits 1-3 in 3CEh index 0Dh.

The Ahead B has separate read and write banks in register 3CEh index Dh.

Memory locations:

$C000:$25  5 bytes  'AHEAD'

ID Ahead chipset:

portw[$3CE]:=$200f;

case port[$3CF] of

$20:Ahead A (with 512k ??)

$21:Ahead B (with 1M ??)

end;

Modes:

22h  T   132   44  16 (8x8)

23h  T   132   25  16 (8x14)

24h  T   132   28  16 (8x)

25h  G   640  480  16 planar

26h  G   640  480  16 planar

2Fh  T   160   50  16

34h  T    80   66  16

50h  T   132   25   2

52h  T   132   44   2

60h  G   640  400 256 packed

61h  G   640  480 256 packed

62h  G   800  600 256 packed

63h  G  1024  768 256 packed   (Ahead B only)

6Ah  G   800  600  16 planar

70h  G   720  396  16 planar

71h  G   800  600  16 planar

74h  G  1024  768  16 planar

75h  G  1024  768   4

76h  G  1024  768   2

Note: Mode 75h has even bytes in planes 0&2, and odd bytes in planes 1&3.