Trident SuperVGA

Trident 8800BR  512k   Only 128K banks.

8800CS  512k

8900    1M

The Trident 8800 chips have a problem with 256 color modes,

as they always double the pixels output in 256 color mode.

Thus a 640x400 256 color mode (5Ch) actually uses a 1280x400

frame, requiring at least a multi sync monitor.

This problem is fixed on the 8900.

100h (R/W?): Microchannel ID low

bit 0-7  Card ID bit 0-7

101h (R/W?): Microchannel ID high

bit 0-7  Card ID bit 8-15

3C3h (R/W): Microchannel Video Subsystem Enable Register:

bit 0  Enable Microchannel VGA if set

3C4h index  Bh (R): Chip Version

bit 0-3  Chip ID

1 = TR 8800BR

2 = TR 8800CS

3 = TR 8900

4-7  Reserved

Note:  Writing to index Bh selects old mode registers.

Reading from index Bh toggles between old and new

mode registers.

3C4h index  Ch (R/W): Power Up Mode Register 1

3C4h index  Dh (R/W): Old Mode Control 2

bit 0-2  Emulation mode

0=VGA, 3=EGA, 5=CGA,MDA,Hercules

4  Enable Paging mode if set

3C4h index  Dh (R/W): New Mode Control 2

Contains timing info

Note: The old/new Mode Control 1/2 registers are selected by

reading and writing the Chip version register (index Bh).

3C4h index  Eh (R/W): Old Mode Control 1

bit 0  (8900 Only) CRTC Address bit 17 (Show from upper 512kb if set)

1-2  128kb Bank number (0-3)

3  16 bit interface if set

3C4h index  Eh (R/W): New Mode Control 1

bit 0-3  64k Bank nr. For write ops. xor bank number with 2,

for read ops use bank number directly.

4-7  Reserved

Note: The old/new Mode Control 1/2 registers are selected by

reading and writing the Chip version register (index Bh).

3C4h index  Fh (R/W): Power-up Mode 2

bit 0-3  Switch settings

4  Bus type

5  I/O address 3xx or 2xx

6  Enable ON-Card ROM if set

7  16 bit ROM access if set

3d4h index 1Eh (R/W): Module Testing Register

bit 2  Vertical interlace if set

5  CRTC starting address bit 16

7  (8900 Only) Host address bit 16.

Enables 128k CRTC address if set

3d4h index 1Fh (R/W): Software Programming Register  (8900 Only)

bit 0-1  Memory size  0=256k, 1=512k, 2=768k, 3=1M.

3d4h index 1Fh (R/W): Scratch Register               (8800 Only)

bit 0  Paged memory mode in effect

1  Memory size  0=256k, 1=512k

2  Analog monitor attached

3  44.9 MHz oscilator present

Note: This register is set by software.

3d4h index 22h (R): CPU Latch Read Back

bit 0-7  Data Latch value for current read plane.

3d4h index 24h (R): Attribute State Read Back

bit 0-6  Reserved

7  Attribute Controller State

If set the next write to 3C0h will go to the data

register, if clear to the index register.

3d4h index 26h (R): Attribute Index Read Back

bit 0-7  Attribute Index Register value

46E8h (R):  Video Subsystem Enable Register

bit 3  Enable VGA if set

Bank selection:

Trident VGAs can operate in 2 different modes:

Old Mode, with a 128k window to display memory at A000h - BFFFh

and New Mode, with a 64k window to display memory at A000h - AFFFh.

Old/New mode is selected by reading/writing the Chip Version Register

(3C4h index 0Bh).

Each mode has its own registers at 3C4h index 0Dh and 0Eh.

ID Trident VGA:

port[$3c4]:=14;

old_value:=port[$3c5];

port[$3c5]:=0;

value:=port[$3c5] and 15;

port[$3c5]:=old_value;

if value=2 then TRIDENT VGA  !!!

ID which Trident:

portw[$3c4]:=11;     {Force old definitions}

if port[$3c5]>=3 then TRIDENT_8900 !!!

else TRIDENT_8800 !!!

Video Modes:

50h  T    80   30  16  (8x16)

51h  T    80   43  16  (8x11)

52h  T    80   60  16  (8x8)

53h  T   132   25  16  (8x14)

54h  T   132   30  16  (8x16)

55h  T   132   43  16  (8x11)

56h  T   132   60  16  (8x8)

57h  T   132   25  16  (9x14)

58h  T   132   30  16  (9x16)

59h  T   132   43  16  (9x11)

5Ah  T   132   60  16  (9x8)

5Bh  G   800  600  16  planar

5Ch  G   640  400 256  packed

5Dh  G   640  480 256  packed

5Eh  G   800  600 256  packed           (Undocumented on 8800)

5Fh  G  1024  768  16  planar

60h  G  1024  768   4                   8900 Only

61h  G   768 1024  16  planar

62h  G  1024  768 256  packed           8900 Only

ZyMOS POACH51 modes:

60h  G   960  720  16  planar

61h  G  1280  640  16  planar

62h  G   512  512 256  packed

63h  G   720  540  16  planar

64h  G   720  540 256  packed

6Ah  G   800  600  16  planar

Everex Viewpoint use Everex modes.

Bios extensions:

----------1000-------------------------------

INT 10 - VIDEO - SET VIDEO MODE

....

Return: AH = Status of call:  (Trident Super VGA Chips)

Trident 8800            Trident 8900

00h   Succesfull                do

80h   Fail. Wrong switch        do

81h   Insufficient Video        do

Memory.

82h   The 36MHz crystal        Mode not supported

cannot support the

mode

83h   The 40MHz crystal        Mode not supported

cannot support the

mode.

84h   The 44.9MHz crystal      Mode not supported

cannot support the

mode.

85h   Dead or no crystal

86h                            Wrong CRTC base for dual screen

87h                            Text mode not supported